# MR Exposure SR

## SUBMITTED BY Steven Nichols / Joshua Trzasko

## On Behalf of Working Group 16

(Magnetic Resonance)

## Introduction/Scope

## This work item proposes two DICOM Structured Report IODs to represent patient-specific MR output conditions defined prior to the MR examination, the output conditions enforced by the MR system during the examination, and the resulting system-calculated exposure metrics (e.g., SAR, B1+rms, dB/dt). The objective is to enable interoperable exchange of RF and gradient output constraints (e.g., SAR, B1+rms, dB/dt) specified due to implants, comorbidities, or research protocols, and to record the enforced conditions and resulting exposure metrics for downstream safety analysis and research.

## The proposal extends existing DICOM safety work, including the Implant Safety attributes proposed in CP-1992, by providing a plan-versus-performed model analogous to Imaging Agent Administration Reporting (Sup 164)

## Limitations of Current Standard

## CP-1992 introduces instance-level attributes to record MR safety constraints that influenced image acquisition, such as SAR and B1+rms limits. While valuable, this approach has limitations:

- It does not capture clinical rationale for the specified exposure conditions.

- It cannot represent pre-examination planning performed by protocoling systems or MR safety services.

- It fragments exposure information across instances, complicating study-level analysis.

- It does not support workflows where prescribed limits differ from scanner defaults, including off-label or investigational use.

- TODO add methods of calculations, assumptions and considerations for anatomical regions, etc..

## WG-16 discussions indicate that CP-1992 remains in work, and several advocates prefer a study-level SR approach rather than an image-instance-centric model, particularly for safety analytics and research use.

## The absence of a standard has led to non-interoperable, non-queryable safety data, limiting post-market surveillance, multicenter research, and real-world evidence generation.

## Description of Proposal

## This proposal defines two new SR IODs:

- Planned MR Exposure Conditions SR IOD
Represents patient-specific MR output conditions and associated exposure metrics defined prior to the MR examination.

- Performed MR Exposure SR IOD
Documents the MR output conditions enforced during the MR examination and the corresponding system-calculated exposure metrics achieved, and references the associated planned conditions when applicable.

## Planned MR Exposure Conditions SR IOD

- Authored by a clinician, physicist, MR safety service, or protocoling system.

- May represent standard-of-care, off-label, or research-defined exposure conditions.

- Encodes specified output conditions and associated limits such as:

  - RF (Whole-body SAR, Head SAR, B1+rms, B1+peak)

  - Gradient (dB/dt, maximum gradient slew rate)

  - Static field (spatial field gradient (SFG), where applicable)

  - Maximum scan or sequence duration constraints

  - Operating Mode (per RF, gradient, etc.)

- Includes clinical rationale, implant labeling, published guidelines, or risk-benefit justification.

## Performed MR Exposure SR IOD

## Key characteristics:

- Generated by the MR system under operator control.

- Records the output conditions enforced during the MR examination, including applicable operating mode and associated limits.

- References the Planned MR Exposure Conditions SR IOD when applicable.

- Documents deviations between planned conditions and enforced conditions due to system constraints, safety controls, or operator intervention.

- Supports aggregation across multiple series and scanners within a study.

## This object provides an authoritative record of real-world MR exposure constraints.

## Parts of Standard Affected

This work item will affect Parts 2, 3, 4, 5, 6 and 16 of the DICOM standard.

## Resources & Timeline

- Editor: Steve Nichols (GEHC) and Joshua Trzasko, D, PhD (Mayo Clinic), have volunteered to edit the supplement.

- Working Group Support: WG-16 has indicated support for the work.

- Estimated Duration: 4 to 6 WG-06 review cycles.

- WG-06 Time: Approximately 4 hours per cycle to review early draft, public comment, letter ballot, and final text.

- TODO: ACR pediatric working group, AAPM, IEC, AdvaMed MR group